LANE DETECTION SYSTEM

Project Documentation

ADAS Engineering Portfolio Project

Technologies: Python  |  OpenCV  |  NumPy

# 1. Project Overview

This project implements a real-time lane detection system using Python and OpenCV. The system processes dashcam video footage to detect lane markings, extrapolate full-length lane lines, and overlay a transparent green fill between the detected lanes — similar to production ADAS (Advanced Driver Assistance Systems) used in modern vehicles.

The project was built step by step, starting from reading a single video frame and progressively adding grayscale conversion, edge detection, masking, line detection, extrapolation, and lane fill — testing and fixing issues at every stage.

Key Capabilities

- Real-time lane detection on dashcam video at 1280x720 resolution

- Polygon masking to isolate the road region of interest

- Hough Transform based line detection with slope-based left/right separation

- Line extrapolation using slope-intercept math for full-length lane lines

- Temporal smoothing using last known good line memory for stable detection through dashed lane gaps

- Transparent green lane fill between detected boundaries

# 2. Technology Stack

| Library | Version | Purpose |
| --- | --- | --- |
| Python | 3.14.5 | Core programming language |
| OpenCV (cv2) | 4.13.0 | Computer vision — image processing, edge detection, line detection |
| NumPy | Latest | Array operations, masking, mathematical calculations |

# 3. How It Works — The Pipeline

The system processes every frame of the video through a 6-step pipeline:

| Step | Function | What It Does |
| --- | --- | --- |
| 1 | cv2.cvtColor() | Convert colour frame to grayscale — edge detection works on brightness not colour |
| 2 | cv2.GaussianBlur() | Smooth the image to remove random pixel noise — prevents false edge detection |
| 3 | cv2.Canny() | Detect edges — find all pixels where brightness changes suddenly (lane markings) |
| 4 | cv2.fillPoly() + bitwise_and() | Apply trapezoid mask — keep only edges inside the road area, remove sky and surroundings |
| 5 | cv2.HoughLinesP() | Find straight lines from edge pixels — returns start and end coordinates of each line |
| 6 | extrapolate_line() + cv2.line() | Average and extend detected segments into full-length lane lines, draw on frame |

# 4. Key Technical Concepts

## 4.1 Region of Interest Masking

A major challenge in lane detection is that the camera captures more than just the road — sky, trees, signs, and surrounding vehicles all produce edges that confuse the line detector. A trapezoid-shaped mask is applied after edge detection to keep only edges inside the road area.

The trapezoid coordinates were determined by overlaying a pixel grid on the video frame and reading the coordinates of the road boundaries:

polygon = np.array([[(200, 580), (480, 320), (700, 320), (1100, 580)]])

These coordinates are specific to this video at 1280x720 resolution. A different video or resolution requires re-tuning these values.

## 4.2 Slope-Based Lane Separation

HoughLinesP detects all straight lines in the masked image — including lines from road markings, barriers, and noise. The slope of each line is calculated to separate left lane lines from right lane lines:

slope = (y2 - y1) / (x2 - x1)

In image coordinates, y increases downward. Therefore:

- Negative slope (< -0.3) = left lane line — goes from bottom-left to top-right

- Positive slope (> +0.3) = right lane line — goes from bottom-right to top-left

- Near-zero slope (-0.3 to +0.3) = horizontal road noise — ignored

## 4.3 Line Extrapolation

HoughLinesP returns short fragmented line segments — not full-length lane lines. The extrapolate_line() function averages all detected segments for each side into one representative line using the slope-intercept equation, then extends it from the bottom of the frame to the horizon:

# y = slope * x + intercept

# rearranged: x = (y - intercept) / slope

x_bottom = int((y_bottom - intercept) / slope)

x_top    = int((y_top    - intercept) / slope)

## 4.4 Temporal Smoothing

Dashed lane markings create gaps where no lane line is detected in a given frame. Without handling this, the overlay flickers on and off. The system stores the last successfully detected line for each side and uses it when the current frame has no detection:

if left_lines:

last_left = extrapolate_line(left_lines, y_bottom, y_top)

# if no left lines detected this frame,

# last_left keeps its previous value — no flicker

## 4.5 Transparent Lane Fill

A transparent green overlay is drawn between the two detected lane lines using cv2.addWeighted() to blend a solid fill with the original frame:

overlay = frame.copy()

cv2.fillPoly(overlay, lane_polygon, (0, 255, 0))

cv2.addWeighted(overlay, 0.3, frame, 0.7, 0, frame)

The 0.3 weight gives 30% green opacity — enough to clearly mark the lane without obscuring the road view.

# 5. Problems Faced and Solutions

| Problem | Cause | Solution |
| --- | --- | --- |
| Video too slow to process | Original video was 4K (3840x2160) | Resized every frame to 1280x720 using cv2.resize() |
| Hood of car detected as lane | Polygon mask included the hood area | Added cv2.rectangle() to block the bottom portion of the frame before masking |
| Too many false edge detections | Low Canny thresholds picking up road texture noise | Increased thresholds from (50,150) to (120,240) to only detect strong edges |
| Lane fill creating wrong shape (bowtie) | fillPoly points were in wrong order — jumping across the shape | Reordered points to go around the polygon: bottom-left, top-left, top-right, bottom-right |
| Left and right lines mixed up | slope threshold too loose — nearly horizontal lines included | Tightened slope threshold to 0.3 — only lines steeper than 30 degrees accepted |
| Lines flickering on dashed lanes | No detection in gap frames caused line to disappear | Added last_left and last_right memory variables to hold last good detection |
| Lines converging to a triangle at top | Extrapolation extending lines to vanishing point | Added min_half_width constraint to force minimum gap between lines at top |
| Green lines appearing on hood | y coordinate check missing before drawing | Added if y1 < 430 and y2 < 430 check to only draw lines above the hood |
| Video filename wrong | File saved as road.mp4.mp4 (double extension) | Renamed file to remove duplicate extension |

# 6. Final Code

The complete working implementation:

import cv2

import numpy as np

def extrapolate_line(lines, y_bottom, y_top):

slopes, intercepts = [], []

for x1, y1, x2, y2 in lines:

if x2 - x1 == 0:

continue

slope = (y2 - y1) / (x2 - x1)

intercept = y1 - slope * x1

slopes.append(slope)

intercepts.append(intercept)

if not slopes:

return None

slope     = np.mean(slopes)

intercept = np.mean(intercepts)

x_bottom  = int((y_bottom - intercept) / slope)

x_top     = int((y_top    - intercept) / slope)

return (x_bottom, y_bottom, x_top, y_top)

cap        = cv2.VideoCapture('lane.mp4')

last_left  = None

last_right = None

while True:

ret, frame = cap.read()

if not ret: break

frame   = cv2.resize(frame, (1280, 720))

gray    = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

blurred = cv2.GaussianBlur(gray, (5, 5), 0)

edges   = cv2.Canny(blurred, 120, 240)

polygon = np.array([[(200,580),(480,320),(700,320),(1100,580)]])

mask    = np.zeros_like(edges)

cv2.fillPoly(mask, polygon, 255)

masked_edges = cv2.bitwise_and(edges, mask)

lines = cv2.HoughLinesP(masked_edges, 1, np.pi/180,

threshold=50, minLineLength=40, maxLineGap=5)

left_lines, right_lines = [], []

if lines is not None:

for line in lines:

x1, y1, x2, y2 = line[0]

if x2 - x1 == 0: continue

slope = (y2 - y1) / (x2 - x1)

if slope < -0.3: left_lines.append((x1,y1,x2,y2))

elif slope > 0.3: right_lines.append((x1,y1,x2,y2))

if left_lines:  last_left  = extrapolate_line(left_lines,  580, 330)

if right_lines: last_right = extrapolate_line(right_lines, 580, 330)

if last_left and last_right:

lx1,ly1,lx2,ly2 = last_left

rx1,ry1,rx2,ry2 = last_right

lane_polygon = np.array([[(lx1,ly1),(lx2,ly2),(rx2,ry2),(rx1,ry1)]])

overlay = frame.copy()

cv2.fillPoly(overlay, lane_polygon, (0, 255, 0))

cv2.addWeighted(overlay, 0.3, frame, 0.7, 0, frame)

cv2.line(frame, (lx1,ly1), (lx2,ly2), (0,255,0), 4)

cv2.line(frame, (rx1,ry1), (rx2,ry2), (0,255,0), 4)

cv2.imshow('Lane Detection', frame)

if cv2.waitKey(1) == 27: break

cap.release()

cv2.destroyAllWindows()

# 7. Known Limitations

Honest failure analysis — this is what makes a strong engineering portfolio:

- Sharp curves — the system assumes straight lane lines. On curved roads the extrapolated straight lines deviate significantly from the actual lane.

- Low light and night driving — Canny edge detection relies on brightness contrast. In poor lighting the lane markings produce weak edges that fall below the detection threshold.

- Rain and wet roads — reflections on wet roads create false edges that confuse the slope-based line separator.

- Faded or worn lane markings — old or partially visible lane lines don't produce enough edge pixels to meet the HoughLinesP threshold.

- Lane changes — when the car changes lanes, the polygon mask no longer aligns correctly and detection breaks temporarily.

- Video-specific polygon — the trapezoid coordinates are tuned for this specific video and camera angle. A different dashcam requires re-tuning.

# 8. What I Learned

- How images are stored as NumPy arrays and how every pixel is just a number

- The full OpenCV computer vision pipeline — read, grayscale, blur, edge detect, mask, detect lines, draw

- How to define a region of interest using polygon masking and bitwise operations

- Slope and intercept mathematics for characterising and extending straight lines

- How to use HoughLinesP parameters (threshold, minLineLength, maxLineGap) and when to tune each

- Temporal smoothing using memory variables to handle detection gaps in dashed lane markings

- How cv2.addWeighted() creates transparent overlays by blending two frames

- How to debug computer vision output by showing intermediate pipeline stages

- How to tune detection parameters iteratively by observing real output rather than guessing

# 9. Future Improvements

- Lane departure warning — detect when the car drifts toward a lane boundary and trigger an alert

- Curved lane detection — replace straight line extrapolation with polynomial fitting for curved roads

- Night mode — adaptive Canny thresholds that automatically adjust based on average frame brightness

- FPS counter — display processing speed on screen to measure performance

- Multi-lane detection — extend the system to detect all visible lanes, not just the immediate left and right boundaries

- Deep learning upgrade — replace HoughLinesP with a neural network lane detector for more robust detection in difficult conditions

# 10. How to Run

Requirements

pip install opencv-python numpy

Setup

- Place lane.mp4 in the same folder as lane.py

- Video should be dashcam footage with visible lane markings

- If using a different video, re-tune the polygon coordinates using the grid method

Run

python lane.py

Controls

- Press Escape to stop the video and close the window